Task  # 11

Name : Nabiha Sabir

Roll no : bssem-s24-019

Section :5A

Subject :Computer Network  lab

Submitted to : mam

Question 1: What is an ACL in Computer Networks?

ACL stands for Access Control List. It is a set of rules configured on a router or switch that controls which network traffic is allowed to pass through and which traffic is blocked.

Every packet that enters or exits a router interface is checked against the ACL rules. The router reads the rules from top to bottom and applies the first matching rule to the packet.

In simple words, an ACL works like a filter. It looks at incoming or outgoing packets and decides whether to allow them or drop them based on the defined rules.

Question 2: Why Do We Use ACL in Networking?

ACLs are used in networking for the following reasons:

Security: ACLs prevent unauthorized users or devices from accessing sensitive parts of the network.

Traffic Filtering: ACLs block unwanted traffic from entering or leaving a network segment.

Network Policy Enforcement: Network administrators use ACLs to enforce company policies, such as preventing certain departments from accessing specific servers.

Protection of Resources: ACLs protect critical servers and devices by allowing only permitted hosts to communicate with them.

Control Over Services: ACLs can block or allow specific protocols and services such as HTTP, FTP, or Telnet.

Question 3: Difference Between Permit and Deny in ACL

| Feature | Permit | Deny |
| --- | --- | --- |
| Function | Allows the packet to pass through | Drops the packet and blocks it |
| Effect on Traffic | Traffic is forwarded normally | Traffic is discarded |
| Usage | Used to allow specific hosts or networks | Used to block specific hosts or networks |
| Default Behavior | Must be explicitly written | An implicit deny all exists at the end of every ACL |

Important Note:

Every ACL has an implicit "deny any" statement at the end. This means if a packet does not match any rule in the ACL, it is automatically denied. Because of this, every ACL must have at least one permit statement, otherwise all traffic will be blocked.

Question 4: Difference Between Standard ACL and Extended ACL

| Feature | Standard ACL | Extended ACL |
| --- | --- | --- |
| ACL Number Range | 1 to 99 | 100 to 199 |
| Filters Based On | Source IP address only | Source IP, Destination IP, Protocol, and Port number |
| Flexibility | Less flexible | More flexible and precise |
| Placement Rule | Placed close to the destination | Placed close to the source |
| Use Case | Block an entire host or network | Block a specific service like HTTP or FTP from a specific host |
| Example | Block all traffic from 192.168.1.10 | Block 192.168.1.10 from accessing port 80 on a server |

Question 5: Standard ACL Configuration on Cisco Packet Tracer

Network Details:

- Network A: 192.168.1.0/24 connected to R1

- Network B: 192.168.2.0/24 connected to R2

- Network C: 192.168.3.0/24 connected to R3

- PC to block: 192.168.1.10

- Requirement: Block 192.168.1.10 from accessing Network C, allow all others

Topology:

Network A          Network B           Network C

192.168.1.0  ---  192.168.2.0  ---  192.168.3.0

|                  |                  |

R1 --------------- R2 --------------- R3

|

PC 192.168.1.10  <-- BLOCKED from Network C

Other PCs        <-- ALLOWED

Interface IP Addressing Plan:

| Router | Interface | IP Address |
| --- | --- | --- |
| R1 | Fa0/0 | 192.168.1.1 / 24 |
| R1 | Se0/0/0 | 192.168.12.1 / 24 |
| R2 | Se0/0/0 | 192.168.12.2 / 24 |
| R2 | Se0/0/1 | 192.168.23.1 / 24 |
| R3 | Se0/0/0 | 192.168.23.2 / 24 |
| R3 | Fa0/0 | 192.168.3.1 / 24 |

Step 1: Configure R1

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# interface fa0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface se0/0/0

R1(config-if)# ip address 192.168.12.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# ip route 192.168.2.0 255.255.255.0 192.168.12.2

R1(config)# ip route 192.168.3.0 255.255.255.0 192.168.12.2

R1(config)# end

R1# write memory

Step 2: Configure R2

Router> enable

Router# configure terminal

Router(config)# hostname R2

R2(config)# interface se0/0/0

R2(config-if)# ip address 192.168.12.2 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface se0/0/1

R2(config-if)# ip address 192.168.23.1 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# ip route 192.168.1.0 255.255.255.0 192.168.12.1

R2(config)# ip route 192.168.3.0 255.255.255.0 192.168.23.2

R2(config)# end

R2# write memory

Step 3: Configure R3

Router> enable

Router# configure terminal

Router(config)# hostname R3

R3(config)# interface se0/0/0

R3(config-if)# ip address 192.168.23.2 255.255.255.0

R3(config-if)# no shutdown

R3(config-if)# exit

R3(config)# interface fa0/0

R3(config-if)# ip address 192.168.3.1 255.255.255.0

R3(config-if)# no shutdown

R3(config-if)# exit

R3(config)# ip route 192.168.1.0 255.255.255.0 192.168.23.1

R3(config)# ip route 192.168.2.0 255.255.255.0 192.168.23.1

R3(config)# end

R3# write memory

Step 4: Apply Standard ACL on R3

Standard ACL is placed closest to the destination, which is R3 (gateway of Network C).

R3# configure terminal

R3(config)# access-list 10 deny host 192.168.1.10

R3(config)# access-list 10 permit any

R3(config)# interface fa0/0

R3(config-if)# ip access-group 10 out

R3(config-if)# end

R3# write memory

Explanation of ACL Commands:

access-list 10 deny host 192.168.1.10

This rule blocks any packet coming from the specific host 192.168.1.10.

access-list 10 permit any

This rule allows all other traffic to pass through. Without this line, the implicit deny would block all traffic.

ip access-group 10 out

This applies ACL number 10 on the outbound direction of fa0/0, meaning it filters traffic going out toward Network C.

Step 5: Verification Commands

R3# show access-lists

R3# show ip interface fa0/0

R3# show running-config

Final Result:

| Source IP | Destination Network | Result |
| --- | --- | --- |
| 192.168.1.10 | 192.168.3.0 (Network C) | Denied |
| 192.168.1.20 | 192.168.3.0 (Network C) | Allowed |
| 192.168.2.x | 192.168.3.0 (Network C) | Allowed |
| Any host | 192.168.1.0 (Network A) | Allowed |
| Any host | 192.168.2.0 (Network B) | Allowed |