Task  # 12

Name : Nabiha Sabir

Roll no : bssem-s24-019

Section :5A

Subject :Computer Network  lab

Submitted to : mam

Network Topology Overview

Two routers (ISR 1331) connected via a Serial WAN link (Se0/1/0 ↔ Se0/1/0), shown in red.

Left Side — Private Network (192.168.10.0)

| Device | Interface | IP Address |
| --- | --- | --- |
| Router0 | Gig0/0/0 | 192.168.10.10 |
| Router0 | Se0/1/0 | 192.168.30.1 |
| Switch0 (2960-24TT) | Fa0/3 ← Router | — |
| PC0 | Fa0 | 192.168.10.x |
| Server0 | Fa0 | 192.168.10.x |

Right Side — Public Network (192.168.20.0)

| Device | Interface | IP Address |
| --- | --- | --- |
| Router1 | Gig0/0/0 | 192.168.20.10 |
| Router1 | Se0/1/0 | 192.168.30.2 |
| Switch1 (2960-24TT) | Fa0/3 ← Router | — |
| PC1 | Fa0 | 192.168.20.x |
| PC2 | Fa0 | 192.168.20.x |

How NAT Works Here

PC0 (Private: 192.168.10.x)

↓

Switch0

↓

Router0  ← NAT BOUNDARY

[Inside]      [Outside]

192.168.10.10   192.168.30.1

↓  (translates source IP)

WAN Link (192.168.30.0/30)

↓

Router1

↓

Switch1

↓

PC1 / PC2 (Public: 192.168.20.x)

NAT Roles on Router0:

- Inside interface → Gig0/0/0 (faces private 192.168.10.0 network)

- Outside interface → Se0/1/0 (faces WAN / public side)

NAT Translation Process:

- PC0 sends packet with source IP 192.168.10.x

- Router0 translates source IP → 192.168.30.1 (public IP)

- Packet travels over WAN to Router1

- Router1 forwards to PC1/PC2 on 192.168.20.0

- Reply comes back → Router0 reverses translation → delivers to PC0

Key NAT Concepts Illustrated

| Concept | Description |
| --- | --- |
| Inside Local | Private IP of PC0 (192.168.10.x) |
| Inside Global | Public IP used after NAT (192.168.30.1) |
| Outside Global | Destination IP on public side (192.168.20.x) |
| PAT (overload) | Multiple private IPs → one public IP using port numbers |