Task  # 10

Name : Nabiha Sabir

Roll no : bssem-s24-019

Section :5A

Subject :Computer Network  lab

Submitted to : mam

VLAN Configuration and Inter-VLAN Routing for School Network

Network Plan Summary

| Department | VLAN Name | VLAN ID | Network | Default Gateway |
| --- | --- | --- | --- | --- |
| Administration | ADMIN | 10 | 192.168.10.0/24 | 192.168.10.1 |
| Teachers | TEACHERS | 20 | 192.168.20.0/24 | 192.168.20.1 |
| Students | STUDENTS | 30 | 192.168.30.0/24 | 192.168.30.1 |
| Library | LIBRARY | 40 | 192.168.40.0/24 | 192.168.40.1 |

Topology Diagram

ROUTER (Router-on-a-Stick)

|

Fa0/0 (Trunk)

|

SWITCH (2960)

__________|__________

|      |      |      |

Fa0/1  Fa0/2  Fa0/3  Fa0/4

|      |      |      |

PC0    PC1    PC2    PC3

(VLAN10)(VLAN20)(VLAN30)(VLAN40)

PC IP Address Assignment

| PC | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | 10 (ADMIN) | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC1 | 20 (TEACHERS) | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| PC2 | 30 (STUDENTS) | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |
| PC3 | 40 (LIBRARY) | 192.168.40.10 | 255.255.255.0 | 192.168.40.1 |

Step 1: Configure VLANs on the Switch

Switch> enable

Switch# configure terminal

Switch(config)# hostname Switch

Switch(config)# vlan 10

Switch(config-vlan)# name ADMIN

Switch(config-vlan)# exit

Switch(config)# vlan 20

Switch(config-vlan)# name TEACHERS

Switch(config-vlan)# exit

Switch(config)# vlan 30

Switch(config-vlan)# name STUDENTS

Switch(config-vlan)# exit

Switch(config)# vlan 40

Switch(config-vlan)# name LIBRARY

Switch(config-vlan)# exit

Step 2: Assign Switch Ports to VLANs (Access Ports)

Switch(config)# interface fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

Switch(config)# interface fa0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# exit

Switch(config)# interface fa0/3

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 30

Switch(config-if)# exit

Switch(config)# interface fa0/4

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 40

Switch(config-if)# exit

Step 3: Configure Trunk Port on Switch (toward Router)

Switch(config)# interface fa0/5

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 10,20,30,40

Switch(config-if)# exit

Switch(config)# end

Switch# write memory

Step 4: Configure Router Sub-interfaces (Router-on-a-Stick)

Router> enable

Router# configure terminal

Router(config)# hostname Router

Router(config)# interface fa0/0

Router(config-if)# no shutdown

Router(config-if)# exit

Sub-interface for VLAN 10 (ADMIN)

Router(config)# interface fa0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config-subif)# exit

Sub-interface for VLAN 20 (TEACHERS)

Router(config)# interface fa0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

Router(config-subif)# exit

Sub-interface for VLAN 30 (STUDENTS)

Router(config)# interface fa0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.30.1 255.255.255.0

Router(config-subif)# exit

Sub-interface for VLAN 40 (LIBRARY)

Router(config)# interface fa0/0.40

Router(config-subif)# encapsulation dot1Q 40

Router(config-subif)# ip address 192.168.40.1 255.255.255.0

Router(config-subif)# exit

Router(config)# end

Router# write memory

Step 5: Configure IP Addresses on Each PC

Open each PC in Packet Tracer, go to Desktop, then IP Configuration and enter the following:

PC0 (ADMIN - VLAN 10)

IP Address   : 192.168.10.10

Subnet Mask  : 255.255.255.0

Gateway      : 192.168.10.1

PC1 (TEACHERS - VLAN 20)

IP Address   : 192.168.20.10

Subnet Mask  : 255.255.255.0

Gateway      : 192.168.20.1

PC2 (STUDENTS - VLAN 30)

IP Address   : 192.168.30.10

Subnet Mask  : 255.255.255.0

Gateway      : 192.168.30.1

PC3 (LIBRARY - VLAN 40)

IP Address   : 192.168.40.10

Subnet Mask  : 255.255.255.0

Gateway      : 192.168.40.1

Step 6: Verification Commands

On Switch — show vlan brief

Switch# show vlan brief

Expected Output:

VLAN Name                             Status    Ports

---- -------------------------------- --------- ------

1    default                          active

10   ADMIN                            active    Fa0/1

20   TEACHERS                         active    Fa0/2

30   STUDENTS                         active    Fa0/3

40   LIBRARY                          active    Fa0/4

1002 fddi-default                     active

1003 token-ring-default               active

1004 fddinet-default                  active

1005 trnet-default                    active

On Router — show ip interface brief

Router# show ip interface brief

Expected Output:

Interface         IP-Address       OK?   Method   Status                Protocol

FastEthernet0/0   unassigned       YES   unset    up                    up

Fa0/0.10          192.168.10.1     YES   manual   up                    up

Fa0/0.20          192.168.20.1     YES   manual   up                    up

Fa0/0.30          192.168.30.1     YES   manual   up                    up

Fa0/0.40          192.168.40.1     YES   manual   up                    up

Step 7: Test Connectivity — Ping Between VLANs

Open PC0 (VLAN 10) in Packet Tracer, go to Desktop, then Command Prompt and type:

PC0 pinging PC3 (Different VLANs)

C:\> ping 192.168.40.10

Expected Output:

Pinging 192.168.40.10 with 32 bytes of data:

Reply from 192.168.40.10: bytes=32 time=1ms TTL=127

Reply from 192.168.40.10: bytes=32 time=1ms TTL=127

Reply from 192.168.40.10: bytes=32 time=1ms TTL=127

Reply from 192.168.40.10: bytes=32 time=1ms TTL=127

Ping statistics for 192.168.40.10:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

PC1 pinging PC2

C:\> ping 192.168.30.10

Complete Configuration Summary

Switch Final Config

vlan 10

name ADMIN

vlan 20

name TEACHERS

vlan 30

name STUDENTS

vlan 40

name LIBRARY

interface fa0/1

switchport mode access

switchport access vlan 10

interface fa0/2

switchport mode access

switchport access vlan 20

interface fa0/3

switchport mode access

switchport access vlan 30

interface fa0/4

switchport mode access

switchport access vlan 40

interface fa0/5

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40

Router Final Config

interface fa0/0

no shutdown

interface fa0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface fa0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface fa0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

interface fa0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

How Inter-VLAN Routing Works

PC0 (VLAN 10)  sends packet to PC3 (VLAN 40)

|

| Goes to Switch via Fa0/1

|

| Switch forwards to Trunk port Fa0/5 (tagged VLAN 10)

|

| Arrives at Router Fa0/0.10

|

| Router checks destination IP (192.168.40.10)

| Routes packet to Fa0/0.40

|

| Goes back to Switch via Trunk (tagged VLAN 40)

|

| Switch forwards to Fa0/4

|

PC3 (VLAN 40) receives the packet